3000mm 1/2” Super Flexible Cable Jumper

(7/16 DIN Male To 7/16 DIN Female Connectors )

Connector Interface:                            CECC 22190 ;IEC 60169-4 ;DIN 47223

Electrical

Characteristic Impedance                                          50 ohm

Frequency Range                                                DC-3 GHz

VSWR                                                   ≤1.10@698-960MHz

≤1.10@1900-2200MHz

Dielectric Withstanding Voltage                          ≥2700V RMS,50Hz,at sea level

Dielectric Resistance                                              ≥10000MΩ

Intermodulation 3rd                                         ≤ -160dBc@2×20W

Mechanical

Durability                                                 Mating cycles ≥500

Material and Plating

Material                 Plating

Body                        Brass                         Tri-Metal

Insulator                     PTFE                       －

Center conductor              Phosphor bronze                   Ag

Coupling nut                 Brass                                 Ni

Gasket                      Silicone Rubber                        －

Shrinkable Tube               PVC                                －

Environmental

Temperature Range                                           -40℃～+85℃al

Weatherproof Rate                                                IP 67

RoHs 2002/95/EC   Compliant

Cable Family           1/2” Superflexible cable (See below)

1/2″Super  Flexible  Cable

Construction Specification

|  | Material | Diameter(mm) |
| --- | --- | --- |
| 1. Inner Conductor | Copper Clad Aluminum Wire or Copper | 3.55 |
| 2. Dielectric | Physical Foam Polyethylene | 8.70 |
| 3. Outer Conductor | Helical Copper Tube | 12.00 |
| 4. Jacket | Black PE or Low Smoke Halogen-free Fire-retardant PE | 13.40 |

Electrical Characteristics                          Mechanical and Environmental Characteristics

| Capacitance(pF/m) |  |  |  | 80 |  |  |  |  |  | Min. Single Bending Radius(mm) |  |  | 50 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Impedance(ohm) |  |  |  | 50 |  |  |  |  |  | Min. Repeated Bending Radius(mm) |  |  | 125 |  |  |
| Velocity(%) | 83 |  |  |  | No. of Bends |  |  | 15 |  |  |  |  |  |  |  |
| Peak Power Rating(KW) | 19 |  |  |  | Mobile Apply(mm) |  |  | 350 |  |  |  |  |  |  |  |
| RF Peak Voltage(KV) | 1.13 |  |  |  | Bending Moment(N.m) |  |  | 5.0 |  |  |  |  |  |  |  |
| Insulation Resistance(MΩ.km) | >5000 |  |  |  | Tensile Strength(kg) |  |  | 110 |  |  |  |  |  |  |  |
| Cut-off Frequency(GHz) | 12.5 |  |  |  | Storage Temp.(℃) |  |  | -55to+85 |  |  |  |  |  |  |  |
| Insulation Voltage(KV) | 2.5 |  |  |  | Installation Temp. (℃) |  |  | -40to+60 |  |  |  |  |  |  |  |
| Inner Conductor DC Resistance(Ω/km) | 2.85 |  |  |  | Operating Temp.( ℃) |  |  | -55to+85 |  |  |  |  |  |  |  |
| Outer Conductor DC Resistance(Ω/km) |  |  |  | 3.50 |  |  |  |  |  |  |  |  |  |  |  |
| Jacket Spark(KV) |  |  |  | 5.0 |  |  |  |  |  |  |  |  |  |  |  |
| Shielding Effectiveness(dB) | >120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VSWR≤(Return loss≥dB) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0.005-3GHz |  | 1.15 |  | (23) |  |  |  |  |  |  |  |  |  |  |  |
| 0.8-1.0GHz | 1.10 |  | (26) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.7-2.0GHz | 1.10 |  | (26) |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.0-2.4GHz | 1.10 |  | (26) |  |  |  |  |  |  |  |  |  |  |  |  |

Attenuation(VSWR1.0,cable temp.20℃) and Average Power(VSWR1.0,ambient temp.40℃)

| Frequency(MHz) |  | Attenuation(≯dB/100m) |  | Average Power(KW) |  |
| --- | --- | --- | --- | --- | --- |
| 100 |  | 3.22 |  | 3.03 |  |
| 200 | 4.65 |  | 2.11 |  |  |
| 450 | 7.20 |  | 1.37 |  |  |
| 800 | 9.86 |  | 1.00 |  |  |
| 900 | 10.56 |  | 0.94 |  |  |
| 1000 | 11.15 |  | 0.88 |  |  |
| 1500 | 13.80 |  | 0.70 |  |  |
| 1800 | 15.55 |  | 0.63 |  |  |
| 2000 | 16.40 |  | 0.59 |  |  |
| 2200 | 17.35 |  | 0.56 |  |  |
| 2400 | 18.10 |  | 0.53 |  |  |
| 2500 | 18.50 |  | 0.52 |  |  |
| 3000 | 20.90 |  | 0.48 |  |  |