Discharge Tube Type Coaxial Surge protector

| 7/16 Male to 7/16 Female NO. | Name of parts |  | Material (Code) |  |  | Plating |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Connecting Nut |  | Brass rod (HPb59-1) | Tri-metal plating |  |  |
| 2 |  | Shell |  | Brass rod (HPb59-1) | Tri-metal plating |  |  |
| 3 |  | Inner Conductor |  | Brass rod (HPb59-1) | Silver plating |  |  |
| 4 | socket contact |  | Bronze rod(QSn6.5-0.1) |  |  | Silver plating |  |
| 5 | pin contact |  | Brass rod (HPb59-1) |  |  | Silver plating |  |
| 6 | Airproof Gasket |  | silicone rubber |  |  | － |  |
| 7 | Insulator |  | polytetrafluorethylene (SFX-1) |  |  | － |  |
| 8 | standard parts |  | Stainless Steel |  |  | － |  |

| Impedance |  |  | 50Ω |
| --- | --- | --- | --- |
| Contact resistance(normality) | Inner conductor |  | ≤0.4mΩ |
|  |  | Outer conductor | ≤0.2mΩ |
| Insulation Resistance (normal) |  |  | ≥1x104MΩ |
| AC(V /min)Withstanding Voltage |  |  | @ 4000V/1min No breakdown and no arcing phenomenon |
| Frequency Range (MHz) |  |  | DC-2600MHz |
| V.S.W.R. |  |  | ≤1.20 |
| Insertion Loss(dB) |  |  | ≤0.25 |
| Rated Power |  |  | 200 |
| Initial discharge voltage(V) |  |  | 90/150/230/350 (Available) |
| Peak discharge voltage(V @1×40μS） |  |  | 1000 |
| Peak discharge current (KA) |  |  | 10/20 |
| Weatherproof Rage |  |  | IP66 |
| Intermodulation(PIM3) |  |  | ≤-155dBc |
| Net Weight (g) |  |  | 290 |