MCX Male Straight Crimp Connector For RG316 Cable

| Interface |  |  |
| --- | --- | --- |
| According to |  | IEC 61169-36;CECC 22220 |
| Electrical |  |  |
| Characteristic Impedance |  | 50 ohm |
| Frequency Range |  | DC-6GHz |
| VSWR |  | ≤1.20@DC-3GHz |
| Dielectric Withstanding Voltage |  | ≥750V RMS,50Hz,at sea level |
| Dielectric Resistance |  | ≥1000MΩ |
| Contact Resistance |  | Center Contact ≤5mΩ |
|  |  | Outer Contact ≤2.5mΩ |
| Mechanical |  |  |
| Durability |  | Mating cycles ≥500 |
| Material and plating |  |  |
|  | Material | Plating |
| Body | Brass | Au |
| Insulator | PTFE | － |
| Center conductor | Phosphor bronze | Au |
| Resilient Contact | Phosphor bronze | Au |
| Ferrule | Annealed Copper | Ni or Au |
| Gasket(O-ring) | Silicone Rubber | － |
| Environmental |  |  |
| Temperature Range |  | -40℃～+85℃ |
| RoHs Compliant |  |  |