MCX Male Right Angle Crimp Connector For RG174 Cable

Interface

According to                                        IEC 61169-36;CECC 22220

Electrical

Characteristic Impedance                                         50 ohm

Frequency Range                                               DC-6GHz

VSWR                                                     ≤1.30@DC-2GHz

Dielectric Withstanding Voltage                      ≥750V RMS,50Hz,at sea level

Dielectric Resistance                                           ≥1000MΩ

Contact Resistance                                     Center Contact ≤5mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                            Mating cycles  ≥500

Material and plating

Material               Plating

Body                         Brass                         Au

Insulator                      PTFE                        －

Center conductor                Phosphor bronze                     Au

Resilient Contact                Phosphor bronze                     Au

Ferrule                        Annealed Copper                     Ni

Gasket(O-ring)                  Silicone Rubber                      －

Environmental

Temperature Range                                            -40℃～+85℃

RoHs Compliant