MMCX Male Crimp Connector For RG316 Cable

Interface

According to                                              CECC 22340

Electrical

Characteristic Impedance                                         50 ohm

Frequency Range                                               DC-6GHz

VSWR                                                    ≤1.20@DC-3GHz

Dielectric Withstanding Voltage                    ≥500V RMS,50Hz,at sea level

Dielectric Resistance                                            ≥1000MΩ

Contact Resistance                                    Center Contact ≤5mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                             Mating cycles≥500

Material and plating

Material              Plating

Body                         Brass                        Au

Insulator                      PTFE                      －

Center conductor               Phosphor bronze                     Au

Resilient Contact               Phosphor bronze                     Au

Ferrule                       Annealed Copper                     Ni or Au

Environmental

Temperature Range                                           -40℃～+85℃

RoHs Compliant