# MMCX Male to MMCX Male Adaptor

Interface According to CECC 22340 Electrical Characteristic Impedance 50 ohm Frequency Range DC-6GHz VSWR ≤1.10@DC-3GHz Dielectric Withstanding Voltage ≥500V RMS,50Hz,at sea level Dielectric Resistance ≥1000MΩ Contact Resistance Center Contact ≤5mΩ Outer Contact ≤2.5mΩ Mechanical Durability Mating cycles ≥500 Material and plating Material Plating Body Brass Au Insulator PTFE － Center conductor Phosphor bronze Au Resilient Contact Phosphor bronze Au Ferrule Annealed Copper Ni or Au Gasket(O-ring) Silicone Rubber － Environmental Temperature Range -40℃～+85℃ RoHs Compliant.